The Site Book   thesitebook.co.uk	BLANK TEMPLATE

# Confined space entry permit

Blank permit for entry into one confined space. Use it with the risk assessment and safe system of work for the job.

A confined space is an enclosed place, such as a tank, chamber, pit, trench, sewer or well, where being enclosed creates a reasonably foreseeable risk from fire or explosion, overheating, gas, fume, vapour or lack of oxygen, rising liquid, or free-flowing solids.

Only enter if it is not reasonably practicable to do the work without entering. If entry is needed, work to a safe system of work, and have rescue arrangements ready before anyone goes in.

A permit is usually needed where there is a reasonably foreseeable risk of serious injury in entering or working in the space. It records the checks and authorisations. It does not make the job safe by itself.

# The space and the work

| Permit number |  | Site address |  |
| --- | --- | --- | --- |
| Confined space and location |  | Work to be done |  |
| Risk assessment reference |  | Other permits in force |  |
| Supervisor |  | Maximum time per entry |  |
| Valid from (date and time) |  | Valid until (date and time) |  |

Hazards identified    □ Lack of oxygen    □ Toxic gas, fume or vapour    □ Flammable atmosphere    □ Heat    □ Rising liquid    □ Free-flowing solid    □ Other

# Precautions before entry

Tick each precaution that is in place, or N/A where it is not needed. Nobody enters until every precaution needed is in place.

| Precaution | In place | N/A | Checked by |
| --- | --- | --- | --- |
| Isolated from gases, liquids and other flowing materials |  |  |  |
| Mechanical and electrical equipment isolated and locked off |  |  |  |
| Residues removed, and the space cleaned or purged |  |  |  |
| Ventilation in place |  |  |  |
| Atmosphere tested before entry (results below) |  |  |  |
| Communication agreed between people inside and outside, including how to raise the alarm |  |  |  |
| Safe way in and out |  |  |  |
| Suitable equipment, lighting, PPE and RPE |  |  |  |
| Fire prevention, including no smoking |  |  |  |
| Everyone entering trained and competent for this work |  |  |  |
| Rescue arrangements and equipment ready (see below) |  |  |  |

# Atmosphere tests

Test the atmosphere before entry. Retest or monitor it during the work where the risk assessment says so. Record the results.

| Date and time | Where tested | Oxygen (%) | Flammable (% LEL) | Toxic gas and reading | Tested by |
| --- | --- | --- | --- | --- | --- |

# Emergency and rescue

Rescue arrangements must be in place before anyone enters, with the equipment and trained people to carry them out. Do not rely on the emergency services. Never go in to help someone who has collapsed unless you are protected from the same danger.

| Rescue method |  | Rescue equipment at the entry point |  |
| --- | --- | --- | --- |
| Rescuers (names) |  | Resuscitation and first aid equipment |  |
| How the alarm is raised |  | Site address for the emergency services |  |

# Entry log

Record everyone going in and out of the space.

| Name | Company | Time in | Time out | Time in | Time out |
| --- | --- | --- | --- | --- | --- |

# Issue and acceptance

The issuer signs to confirm the precautions are in place. The person in charge of the work signs to accept the permit, and explains the work, the hazards and the precautions to everyone working under it.

|  | Name | Signature | Date and time |
| --- | --- | --- | --- |
| Issued by |  |  |  |
| Accepted by (person in charge of the work) |  |  |  |

# Extension or shift handover

Before the permit is extended or handed to a new person in charge, check the area is still safe to work in, brief the new person and set a new end time.

| Date and time | New end time | Handed over by | Accepted by |
| --- | --- | --- | --- |

# Hand-back and cancellation

When the work is finished, the person in charge of it signs to hand the permit back. The issuer checks the area and cancels the permit.

|  | Name | Signature | Date and time |
| --- | --- | --- | --- |
| Work finished and area left safe |  |  |  |
| Permit cancelled by issuer |  |  |  |

Based on: The Confined Spaces Regulations 1997, regulations 1, 4 and 5; L101, Safe work in confined spaces (third edition), the Approved Code of Practice and guidance on those Regulations, paragraphs 82 to 98 and 134 to 149; HSE guidance HSG250 on permit-to-work systems.

Confined Spaces Regulations 1997, regulation 1: https://www.legislation.gov.uk/uksi/1997/1713/regulation/1

Confined Spaces Regulations 1997, regulation 4: https://www.legislation.gov.uk/uksi/1997/1713/regulation/4

Confined Spaces Regulations 1997, regulation 5: https://www.legislation.gov.uk/uksi/1997/1713/regulation/5

HSE, Safe work in confined spaces (L101): https://www.hse.gov.uk/pubns/books/l101.htm

HSE, Guidance on permit-to-work systems (HSG250): https://www.hse.gov.uk/pubns/books/hsg250.htm

Blank template from The Site Book, October 2026. Fill it in for the actual job and site.