The Site Book   thesitebook.co.uk	BLANK TEMPLATE

# MEWP pre-use check

Blank sheet for a week of daily pre-use checks on one mobile elevating work platform (MEWP), with a fault log.

The trained operator carries out a basic function check every day before use, following the manufacturer's instructions. Keep a record of the checks.

If you find a fault, isolate the controls, tag the machine and report it to the person in control. Take the MEWP out of service if the fault affects safety.

A MEWP must have a thorough examination by a competent person at least every 6 months, or at the intervals set by an examination scheme drawn up by a competent person. Ask for and keep a copy of the latest report.

Keep the MEWP maintained in efficient working order and good repair.

# The MEWP

| Make, model and fleet or serial number |  | Type (for example scissor lift or boom) |  |
| --- | --- | --- | --- |
| Site |  | Week beginning |  |
| Date of last thorough examination |  | Next thorough examination due |  |
| Operator |  | Operator's training card number and expiry date |  |
| Familiarised on this make and model (date) |  | Maximum wind speed (from the manufacturer) |  |

# Daily pre-use checks

Do the manufacturer's daily checks, then confirm the site checks below before raising the platform.

| Check | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manufacturer's daily function checks done |  |  |  |  |  |  |  |
| Platform and ground controls work, including emergency descent |  |  |  |  |  |  |  |
| Guard rails and toe boards in place |  |  |  |  |  |  |  |
| Ground firm and level, with no trenches, manholes or soft backfill |  |  |  |  |  |  |  |
| Outriggers extended and chocked, with spreader plates if needed |  |  |  |  |  |  |  |
| Area below barriered off against falling objects |  |  |  |  |  |  |  |
| Wind speed within the maximum |  |  |  |  |  |  |  |
| Clear of overhead cables and structures that could trap the operator |  |  |  |  |  |  |  |
| Harness and short lanyard clipped to the basket anchor, where required |  |  |  |  |  |  |  |
| Someone on the ground knows the rescue plan and ground controls |  |  |  |  |  |  |  |
| Checked by (initials) |  |  |  |  |  |  |  |

# Faults reported

| Date | Fault | Reported to | Taken out of use? | Put right (date and by whom) |
| --- | --- | --- | --- | --- |

Based on: HSE, Mobile elevating work platforms (MEWPs); HSE information sheet GEIS6, The selection, management and use of mobile elevating work platforms; the Lifting Operations and Lifting Equipment Regulations 1998, regulation 9; the Provision and Use of Work Equipment Regulations 1998, regulation 5.

HSE, Mobile elevating work platforms (MEWPs): https://www.hse.gov.uk/construction/safetytopics/mewp.htm

HSE, The selection, management and use of mobile elevating work platforms (GEIS6): https://www.hse.gov.uk/pubns/geis6.pdf

Lifting Operations and Lifting Equipment Regulations 1998, regulation 9: https://www.legislation.gov.uk/uksi/1998/2307/regulation/9

Provision and Use of Work Equipment Regulations 1998, regulation 5: https://www.legislation.gov.uk/uksi/1998/2306/regulation/5

Blank template from The Site Book, October 2026. Fill it in for the actual job and site.