The Site Book   thesitebook.co.uk	BLANK TEMPLATE

# Permit to dig

Blank permit for one excavation where there may be underground services.

HSE guidance says a permit system may be appropriate for particularly hazardous work near underground services. Whether or not you use one, digging near underground services needs a safe system of work: plan the work, find and mark the services, and dig safely.

# The work

| Permit number |  | Site address |  |
| --- | --- | --- | --- |
| Location and extent of the dig |  | Work to be done |  |
| Maximum depth |  | Digging method |  |
| Person in charge of the dig |  | Company |  |
| Valid from (date and time) |  | Valid until (date and time) |  |

Sketch of the dig and the services found (or attach a marked-up drawing)

# Information about services

Ask the owners of underground services for plans or other information before work starts. Plans alone are not enough to locate services, and may not show every service connection.

| Service | Plans obtained | Located on site | Marked | Notes |
| --- | --- | --- | --- | --- |
| Electricity |  |  |  |  |
| Gas |  |  |  |  |
| Water |  |  |  |  |
| Telecoms |  |  |  |  |
| Drains and sewers |  |  |  |  |
| Other: |  |  |  |  |

# Locating and marking

Someone with the right experience surveys the whole work area with suitable locating equipment and marks the line of each service found. Use trial holes to confirm a service's position and depth.

| Locating equipment used |  | Survey carried out by |  |
| --- | --- | --- | --- |
| Date of survey |  | Trial holes (number and where) |  |

# Safe digging

Agree the digging method near each service before work starts, then expose services by careful hand digging. Use the locator again often as the dig goes deeper. Never use exposed services as handholds or footholds.

If a cable is struck, the machine driver stays in the cab. Keep everyone away until the cable owner has made it safe.

| Check | Yes | N/A | Notes |
| --- | --- | --- | --- |
| Service plans on site and explained to the dig team |  |  |  |
| Services located and marked on the ground |  |  |  |
| Trial holes confirm position and depth |  |  |  |
| Where a machine digs, someone assists the driver from a safe position |  |  |  |
| Insulated hand tools for digging near electric cables |  |  |  |
| Excavation support, edge protection and a safe way in and out planned |  |  |  |
| Everyone knows what to do if a service is damaged |  |  |  |

# Issue and acceptance

The issuer signs to confirm the precautions are in place. The person in charge of the work signs to accept the permit, and explains the work, the hazards and the precautions to everyone working under it.

|  | Name | Signature | Date and time |
| --- | --- | --- | --- |
| Issued by |  |  |  |
| Accepted by (person in charge of the work) |  |  |  |

# Extension or shift handover

Before the permit is extended or handed to a new person in charge, check the area is still safe to work in, brief the new person and set a new end time.

| Date and time | New end time | Handed over by | Accepted by |
| --- | --- | --- | --- |

# Hand-back and cancellation

Before backfilling, support and protect exposed services, put any warning tape or tiles back above them, and tell the service owner if their plans were wrong.

When the work is finished, the person in charge of it signs to hand the permit back. The issuer checks the area and cancels the permit.

|  | Name | Signature | Date and time |
| --- | --- | --- | --- |
| Work finished and area left safe |  |  |  |
| Permit cancelled by issuer |  |  |  |

Based on: HSE guidance HSG47, Avoiding danger from underground services (third edition), paragraphs 22 to 28, 39, 82, 95 to 115, 128 to 135 and 163 to 166; HSE guidance HSG250 on permit-to-work systems.

HSE, Avoiding danger from underground services (HSG47): https://www.hse.gov.uk/pubns/books/hsg47.htm

HSE, Guidance on permit-to-work systems (HSG250): https://www.hse.gov.uk/pubns/books/hsg250.htm

Blank template from The Site Book, October 2026. Fill it in for the actual job and site.