The Site Book   thesitebook.co.uk	BLANK TEMPLATE

# Roof work permit

Blank permit for controlled access to one roof, for example a fragile roof or a roof on a site that runs a permit system.

Use a permit to control who goes onto a roof and when, for example where the roof is fragile or the client runs a permit-to-work system.

Treat every roof as fragile until a competent person has confirmed it is not.

Plan work at height in this order: avoid it where you can, then prevent falls, then reduce the distance and consequences of any fall.

# The work

| Permit number |  | Site address |  |
| --- | --- | --- | --- |
| Roof and area of work |  | Work to be done |  |
| Roof covering |  | Access route onto the roof |  |
| Person in charge of the work |  | Company |  |
| Valid from (date and time) |  | Valid until (date and time) |  |

Roof surface    □ Fragile    □ Has fragile roof lights or areas    □ Confirmed non-fragile by a competent person

# Precautions before access

Tick each precaution that is in place, or N/A where it is not needed. Nobody goes onto the roof until every precaution needed is in place.

| Check | Yes | N/A | Notes |
| --- | --- | --- | --- |
| Edge protection in place, or other measures to prevent falls agreed |  |  |  |
| Fragile areas and roof lights protected, or a safe area fenced off at least 2 metres from them |  |  |  |
| Staging or platforms on fragile surfaces, with nets or a harness system where needed |  |  |  |
| Warning signs at the access points to fragile roofs |  |  |  |
| People below and the public protected from falling materials |  |  |  |
| Overhead power lines identified and controlled |  |  |  |
| Weather and wind checked, and the roof inspected before work starts |  |  |  |
| Workers have the skills, knowledge and experience for the job, and are supervised |  |  |  |
| Rescue plan and equipment ready |  |  |  |

Only work on the roof when the weather does not put the health or safety of anyone involved at risk.

Have a plan to rescue anyone who falls or is left hanging in a harness. Do not rely on the emergency services.

| Maximum wind speed for this work |  |
| --- | --- |
| Rescue plan and equipment |  |

# Issue and acceptance

The issuer signs to confirm the precautions are in place. The person in charge of the work signs to accept the permit, and explains the work, the hazards and the precautions to everyone working under it.

|  | Name | Signature | Date and time |
| --- | --- | --- | --- |
| Issued by |  |  |  |
| Accepted by (person in charge of the work) |  |  |  |

# Extension or shift handover

Before the permit is extended or handed to a new person in charge, check the area is still safe to work in, brief the new person and set a new end time.

| Date and time | New end time | Handed over by | Accepted by |
| --- | --- | --- | --- |

# Hand-back and cancellation

Before handing the permit back, check everyone is off the roof, remove tools and loose materials, and block or lock off the access when it is not needed.

When the work is finished, the person in charge of it signs to hand the permit back. The issuer checks the area and cancels the permit.

|  | Name | Signature | Date and time |
| --- | --- | --- | --- |
| Work finished and area left safe |  |  |  |
| Permit cancelled by issuer |  |  |  |

Based on: HSE guidance HSG33, Health and safety in roof work (fifth edition), paragraphs 27, 48 to 53, 61 to 64, 80, 124, 180 to 201 and 326; the Work at Height Regulations 2005, regulations 4 and 6; HSE guidance HSG250 on permit-to-work systems.

HSE, Health and safety in roof work (HSG33): https://www.hse.gov.uk/pubns/books/hsg33.htm

Work at Height Regulations 2005, regulation 4: https://www.legislation.gov.uk/uksi/2005/735/regulation/4

Work at Height Regulations 2005, regulation 6: https://www.legislation.gov.uk/uksi/2005/735/regulation/6

HSE, Guidance on permit-to-work systems (HSG250): https://www.hse.gov.uk/pubns/books/hsg250.htm

Blank template from The Site Book, October 2026. Fill it in for the actual job and site.