The Site Book   thesitebook.co.uk	BLANK TEMPLATE

# Scaffold inspection register

Blank register for the inspections of one scaffold, with the details an inspection report must contain and a checklist.

A working platform used for construction work, from which a person could fall 2 metres or more, must not be used unless a competent person has inspected it in position within the previous 7 days.

Also inspect it after it is put up or altered and before it is used, and after anything liable to affect its safety, such as high winds.

The inspector writes a report before the end of the working period and gives it, or a copy, to the person the inspection was for within 24 hours. Keep the report on site until the construction work is finished, then at an office for 3 months.

Record every defect or risk found and the action taken, even when it was put right straight away.

# The scaffold

| Inspection carried out for (name and address) |  | Site and location of the scaffold |  |
| --- | --- | --- | --- |
| Description of the scaffold |  | Scaffold erected by |  |
| Duty rating or loading |  | First inspected before use on (date) |  |

# Inspection record

Reason for the inspection: before first use, the 7-day inspection, after alteration, or after bad weather or another event.

| Date and time | Reason for inspection | Matters that could give rise to a risk | Action taken | Further action needed | Name and position of inspector | Signature |
| --- | --- | --- | --- | --- | --- | --- |

# Inspection checklist

Use the checklist at each inspection. Tick each item that is in order and record anything else in the register.

| Check | Date: | Date: | Date: | Date: | Date: | Date: |
| --- | --- | --- | --- | --- | --- | --- |
| Base firm and level, and not undermined |  |  |  |  |  |  |
| Braced and tied as designed |  |  |  |  |  |  |
| Platforms fully boarded and wide enough |  |  |  |  |  |  |
| Boards supported, without excessive overhang |  |  |  |  |  |  |
| Guard rails and toe boards in place |  |  |  |  |  |  |
| Safe access to the platforms |  |  |  |  |  |  |
| Loading bays have fall protection |  |  |  |  |  |  |
| Sheeting or netting only where the scaffold is designed for it |  |  |  |  |  |  |
| Incomplete parts signed and blocked off |  |  |  |  |  |  |
| Inspected by (initials) |  |  |  |  |  |  |

Based on: The Work at Height Regulations 2005, regulation 12 and Schedule 7 (the particulars an inspection report must contain); HSE, Scaffolds.

Work at Height Regulations 2005, regulation 12: https://www.legislation.gov.uk/uksi/2005/735/regulation/12

Work at Height Regulations 2005, Schedule 7: https://www.legislation.gov.uk/uksi/2005/735/schedule/7

HSE, Scaffolds: https://www.hse.gov.uk/construction/safetytopics/scaffoldinginfo.htm

Blank template from The Site Book, October 2026. Fill it in for the actual job and site.