The Site Book   thesitebook.co.uk	BLANK TEMPLATE

# Christmas shutdown site security checklist

Before the site closes for the holidays. Tick each line, or note why it does not apply.

| Site |  | Checked by |  |
| --- | --- | --- | --- |
| Last working day |  | Restart date |  |

| Emergency contact (name and phone) |  |
| --- | --- |
| Site checks during shutdown by |  |

| Item | Done | Notes |
| --- | --- | --- |
| Shutdown and restart dates are agreed, and it's clear who holds the keys and who is the emergency contact while the site is closed. |  |  |
| The client, the neighbours who need to know and any alarm or security company have the dates and the emergency contact number. |  |  |
| Perimeter fencing or hoarding is complete, gaps at gates and where fencing meets other structures are closed, and gates are locked. (CDM reg 18(2); HSE) |  |  |
| Materials, skips and plant are kept away from the fence line so they can't be used to climb in. (HSE) |  |  |
| Bottom-lift ladders are removed from scaffolds and stored securely, or a locked ladder guard is fitted. Hinged ladder gates and trap doors are padlocked. (HSE) |  |  |
| Ladders are removed from excavations. Excavations, pits, manholes, stairwells and open edges are covered or barriered off. (HSE) |  |  |
| Plant and vehicles are isolated and immobilised, keys are removed, and they are locked in a compound where possible. (HSE) |  |  |
| Hazardous substances, fuel and gas cylinders are locked away. (HSE) |  |  |
| Materials are stacked so they can't topple or roll. Loose sheets, boards, insulation and waste are tied down or removed. (HSE) |  |  |
| Scaffold sheeting, netting and brickguards are secure, and the scaffold contractor knows the site is closing. (HSE) |  |  |
| Temporary works are left stable for the shutdown, and anyone responsible for them has agreed how they are left. (CDM reg 19(2)) |  |  |
| Water is drained down or protected where pipes could freeze, and power is isolated where it isn't needed. |  |  |
| Welfare units are left clean, heaters and cookers are off, and nothing is left charging or switched on that could start a fire. |  |  |
| Tools and valuable materials are taken off site or locked away. |  |  |
| Signs at the entrance give the emergency contact number. Signs alone won't keep children out. (HSE) |  |  |
| Photos of the site are taken on the last day, so damage or a break-in can be spotted on return. |  |  |
| Someone is named to check the site during the shutdown, including after any Met Office weather warning for the area. |  |  |

Site check log during the shutdown (date, who, what was found)

Based on: The Construction (Design and Management) Regulations 2015, regulations 15, 18 and 19; HSE, Protecting the public; HSE, Preventing unauthorised access onto scaffolding and other work platforms; HSE, Temporary works.

CDM 2015, regulation 15: https://www.legislation.gov.uk/uksi/2015/51/regulation/15

CDM 2015, regulation 18: https://www.legislation.gov.uk/uksi/2015/51/regulation/18

CDM 2015, regulation 19: https://www.legislation.gov.uk/uksi/2015/51/regulation/19

HSE, Protecting the public: https://www.hse.gov.uk/construction/safetytopics/publicprotection.htm

HSE, Preventing unauthorised access onto scaffolding and other work platforms: https://www.hse.gov.uk/safetybulletins/ladders-and-scaffold-security.htm

HSE, Temporary works: https://www.hse.gov.uk/construction/safetytopics/temporary-works.htm

Blank template from The Site Book, October 2026. Fill it in for the actual job and site.