The Site Book   thesitebook.co.uk	BLANK TEMPLATE

# January site restart checklist

Before work starts again after the holidays. Tick each line, or note why it does not apply.

| Site |  | Checked by |  |
| --- | --- | --- | --- |
| Restart date |  | Time |  |

| Item | Done | Notes |
| --- | --- | --- |
| Walk the site before anyone starts work. Look for storm damage, break-ins, flooding, frost and anything that has moved. Record what you find. |  |  |
| Every scaffold is inspected by a competent person before use. The last 7-day inspection will be out of date, and high winds call for an inspection too. (WAHR reg 12; HSE) |  |  |
| MEWPs and tower scaffolds are inspected before use after severe weather. (HSE) |  |  |
| Excavations are inspected at the start of the shift and after any event likely to have affected their strength or stability, such as heavy rain or frost. (CDM reg 22(4)) |  |  |
| Temporary works are checked by a competent person before anyone loads them or works near them again. (CDM reg 19(2); HSE) |  |  |
| Thorough examination and inspection dates for lifting equipment and plant are still in date. (LOLER reg 9) |  |  |
| Power, water, heating and welfare are working: toilets, washing, drinking water and a warm rest area. (CDM Sch 2) |  |  |
| Risk assessments and RAMS are reviewed, and updated if the work, sequence, team or conditions have changed. (MHSWR reg 3(3)) |  |  |
| The construction phase plan is updated if the work, design or programme has changed over the break. |  |  |
| Insurance has renewed: employers' liability if you have employees, and any public liability or contract works cover. The current certificates are to hand. (HSE) |  |  |
| Workers' CSCS or other scheme cards are still in date. Check them with CSCS Smart Check, not by eye. |  |  |
| Training and tickets that expired over the break are renewed, for example first aid and plant operator cards. |  |  |
| Anyone new to the site gets a site induction before they start. Everyone else gets a short restart briefing on what has changed. (CDM reg 15(9)) |  |  |
| A restart toolbox talk covers ice, frost, dark mornings, storm damage and anything still out of use. |  |  |
| Emergency arrangements are still right: first aiders on site, contact numbers, the nearest A&E, fire points and the assembly point. |  |  |

Damage or problems found, and what was done

Based on: The Work at Height Regulations 2005, regulation 12; The Construction (Design and Management) Regulations 2015, regulations 15, 19 and 22 and Schedule 2; LOLER 1998, regulation 9; The Management of Health and Safety at Work Regulations 1999, regulation 3; HSE, Scaffolds; HSE, MEWPs; HSE, Temporary works; HSE, Get insurance for your business.

Work at Height Regulations 2005, regulation 12: https://www.legislation.gov.uk/uksi/2005/735/regulation/12

CDM 2015, regulation 15: https://www.legislation.gov.uk/uksi/2015/51/regulation/15

CDM 2015, regulation 19: https://www.legislation.gov.uk/uksi/2015/51/regulation/19

CDM 2015, regulation 22: https://www.legislation.gov.uk/uksi/2015/51/regulation/22

CDM 2015, Schedule 2: https://www.legislation.gov.uk/uksi/2015/51/schedule/2

Lifting Operations and Lifting Equipment Regulations 1998, regulation 9: https://www.legislation.gov.uk/uksi/1998/2307/regulation/9

Management of Health and Safety at Work Regulations 1999, regulation 3: https://www.legislation.gov.uk/uksi/1999/3242/regulation/3

HSE, Scaffolds: https://www.hse.gov.uk/construction/safetytopics/scaffoldinginfo.htm

HSE, Mobile elevating work platforms (MEWPs): https://www.hse.gov.uk/construction/safetytopics/mewp.htm

HSE, Temporary works: https://www.hse.gov.uk/construction/safetytopics/temporary-works.htm

HSE, Get insurance for your business: https://www.hse.gov.uk/simple-health-safety/insurance/index.htm

Blank template from The Site Book, October 2026. Fill it in for the actual job and site.